FLYSHEET AN-26

## REQUIREMENTS FOR PRINTED WIRING BOARDS/FLEX CIRCUITS

The following requirements are imposed for the procurement of printed wiring boards and multilayer flex circuits:

1.	Artwork must be approved by Boeing prior to PWB fabrication.

A.	When Boeing provides pre-approved “Engineering Check Film” with the Gerber Data, the Supplier shall use it for verification of their working tools, or

B.	When Gerber data is utilized (without Boeing “Engineering Check Film”), the Supplier shall deliver to Boeing a full set of Diazo contact copies of the Supplier’s plotted artwork for approval.  Written approval of the Supplier’s Diazo artwork plots will be provided, or

C.	When older designs rely on “Vault Master Artwork”, the Supplier shall verify working tools prior to fabrication.

2.	When specified on the P.O. as Code 2A, Buyer in-process source inspection is required as follows:

A.	Detail stage source inspection unless automated optical inspection is performed or waived by Buyer’s Procurement Quality Engineering responsible personnel.

B.	After etch before soldermask, unless waived by Procurement Quality Engineering.

C.	Boeing source inspection shall verify all first build inner layers on jobs that use the Boeing approved “Engineering Check Film”, and any subsequent artwork changes.

3.	When required by the drawing or specification, all X-ray requirements are after drill prior to electroless copper plate (X-rays must be wet film).

4.	All coupons, cross-sections, PPA specimens, and X-rays (when required), etc., must be traceable to individual printed wiring boards and must accompany each shipment.

A copy of the supplier’s test report analysis for microsections (“Group A”) must accompany the test coupons and microsections shipped to Boeing. (A copy of the shop traveler must be sent along with certifications, when requested.)  Serial numbers must also be on the packer.

Multilayer boards and flex circuits require Buyer’s M&P Lab approval prior to any further assembly operations (i.e. next assy level, heatsinks, etc.).  Test coupons and microsections shall be delivered to Boeing for analysis and approval by Engineering prior to subsequent assembly operations.

When shipments are split, or when the cross-sections are Gang-mounted, note the date of original shipment on the packer.

5.	When PPA test boards are required per lot (reference AN-31 or AA0115-210, para. 3.5.2), ship the PPA board to the receiving location with the first delivery from the lot.

6.	When specified on the P.O. as Code 2A or Code 2, Final Source Inspection shall be performed (single sided, double sided, multilayer, flex, rigid flex) after route and test.

All assets, certifications, P.O. and P.O. changes, drawings, ECN’s, specifications, and shipper shall be submitted at the time boards or flex circuits are presented for source inspection.